PULSAR Version 1.00

Parallel Imaging Utilizing Localized Surface-coil Acquisition and Reconstruction

USER MANUAL

Jim Ji, Swati Rane,

Dan Spence and Jong Bum Son

Magnetic Resonance Systems Lab

Department of Electrical Engineering

Texas A&M University

COPYRIGHT

Copyright 2005, Department of Electrical Engineering, Texas A&M University.

All rights reserved. See the file COPYRIGHT for more information.

You are granted a license to use the Software for academic, research and internal business purposes only, without a fee.  You may distribute the software to third parties provided that the copyright notice and this statement appear on all copies and that no charge is associated with such copies. If you make derivative works and distribute it, you will (1) notify PULSAR developers regarding its distribution of the derivative work, and (2) clearly notify users that such derivative work is a modified version and not the original PULSAR distributed by Texas A&M University.

We MAKE NO REPRESENTATIONS ABOUT THE SUITABILITY OF THIS SOFTWARE FOR ANY PURPOSE.  IT IS PROVIDED "AS IS" WITHOUT EXPRESSED OR IMPLIED WARRANTY. WE SHALL NOT BE LIABLE FOR ANY DAMAGES SUFFERED BY THE USERS OF THIS SOFTWARE.

Any Licensee wishing to make commercial use of the Software should contact:

Jim Ji

Department of Electrical Engineering

Texas A&M University

College Station, TX 77843-3128

jimji@tamu.edu

INDEX

1. Introduction……………………………………………………………………….…...4

2. Quick Start……………………………………………………………………….…....4

3. Organization and Structure of Tools…………………………………………….…. 6

3.1 Data Input and Manipulations……………………………………………….….…..7

3.2 Image Reconstruction Process………………………………………………….…..7

3.3 Performance Evaluation………………………………………………………….…8

4. Control Variables……………………………………………………………………...8

4.1 Variables……………………………………………………………………………8

5. Individual Subroutines……………………………………………………………....11

6. Structures…………………………………………………………………………......15

USER MANUAL

Jim X. Ji, Swati D. Rane

Department of Electrical Engineering, Texas A&M University

1. Introduction

This manual is written with the first-time user in mind, to explain how to use this software tool. This software package allows the user to simulate different parallel magnetic resonance imaging techniques such as

1. Sensitivity Encoding (SENSE),

2. Parallel Imaging with Localized Sensitivities (PILS),

3. SiMultaneous Acquisition of Spatial Harmonics (SMASH),

4. GeneRalized Autocalibrating Partially Parallel Acquisition (GRAPPA) and

5. Sensitivity Profiles from an Array of Coils for Encoding and Reconstruction In

Parallel (SPACE RIP).

The software tool goes through the entire process of data input, sensitivity estimation and image reconstruction.

The reconstructed image can be used to analyze the performance of the reconstruction technique by calculating the following parameters:

1. Signal to Noise Ratio (SNR)

2. Artifact Power

3. ‘g’ factor

4. Contrast/ contrast to Noise Ratio (CNR)

5. Resolution Phantom

The entire procedure is broken down into three steps. Each step can be executed individually to analyze the process of parallel magnetic resonance imaging.

Individual functions are defined for every process within the step. The user can replace a function with his/her own function to test the code.

The structure and details of the program execution is provided. A Quick-Start guide is also available.

2. Quick Start

A few example files are available in the ‘workdir/examples’ folder.

Select one of the example files:

1. SENSE1_Inputs_List.m: SENSE from simulated data

2. SENSE2_Inputs_List.m: SENSE for real data (example dataset data1.mat)

3. SENSE3_Inputs_List.m: SENSE from simulated data with a non-linear coil array

for a coronal slice using reference maps.

4. PILS_Inputs_List.m: PILS for real data (example dataset data2.mat)

5. SMASH_Inputs_List.m: SMASH for simulated data

6. GRAPPA_Inputs_List.m: GRAPPA for simulated data

7. GRAPPA2_Inputs_List.m: GRAPPA for real data (Example dataset data1.mat)

with MCMLI reconstruction

8. SPACERIP_Inputs_List.m: SPACERIP for simulated data.

Then, rename and save the file as ‘Inputs_list.m’ in the folder ‘workdir/source/common’. Executing the Run_file.m in ‘workdir/source/common’ will produce an image. The steps in the image reconstruction process can also be viewed in the command window.

For SENSE3_Inputs_List.m, you will be prompted to enter the number of coils in each direction.

E.g.

How many coils in the array along Y axis?

How many coils in the array along X axis?

X is the direction of subsampling.  The values should multiply to equal the number of receiver coils.

For GRAPPA2_Inputs_List.m, you will be prompted to enter the number of columns to be considered.

E.g.,

How many columns to consider for MCMLI reconstruction for GRAPPA?

You can modify example files to try different configurations.

Otherwise follow the following procedure:

- Open the Inputs_list.m file.

- Select a value for data_type from the following options:

1: Complete simulation - image will be selected from the software package itself

2: Use file containing image as input

3: Use full scan kspace data entered as a file

4: Use reduced scan kspace data as a file.

- Enter file containing the data. See Section 3.1 for further details about data input.

- Set all flags to 1.

- If data_type = 1 or data_type = 2, enter all the parameters for the structure Receiver or leave them as is. If data_type = 3 or data_type = 4, let Receiver.sensitvity_estimation=2. Enter a value for Receiver.num_coils.

- If data_type = 4, enter the image’s dimension in Image_size, otherwise leave as is.

- Enter all parameters for the structure Sampling. Values can be kept unchanged but adjust Sampling.center_locs suitably.

- Select a value for option to specify a method for image reconstruction.

- Leave structure Misc unchanged.

- Select a value for per_list. See Section 3.3 for further details about parameter options.

- Save Inputs_list.m.

- Open and run Run_file.m.

- In the case of an error, a message will be displayed in the command window.

E.g. per_list=7

Message in command window:

Error: Variable per_list can take values from 1 to 6. Please correct errors first!

- If there is no error, the command window shows:

Loading files and generating kspace data...

Estimating sensitivity...

Reconstructing image...

- In performance analysis, user will be prompted to select the region of interest and region of noise for SNR measurement. The mouse turns into a cross-hair on the image. Decide on the region and click on the diagonal points (top-left and then bottom-right corners) of the region.

Fig.1: Selecting a region

A rectangle appears showing the region selected, as seen in Figure 1. Repeat procedure for selecting a region of noise.

In the case of other analysis methods, user will be asked whether multiple acquisitions are present and subsequently, to enter the file name. See Section 3.3 for further details about performance evaluation.

- The performance analysis parameter is saved in the structure Perform_parameters. The structure and the reconstructed image are stored in the file parameters.mat.

See Section 3.2 for more information about the image generated, Section 3.3 for more information about recalling values calculated during performance analysis, or Section 4 for more details about variables.

3. Organization and Structure of the Tools

The software can be structured as shown below:

START

Data Input and Manipulation

Image Reconstruction

Performance Analysis

STOP

Fig.2: Broad overview of the software structure

3.1. Data Input and Manipulation

File format:

- Kspace data is entered in the standard MATLAB format as a filename.mat file.

- Each .mat file contains one variable.

The variable name must be the same as the file name.

E.g.

File kspace.mat contains variable kspace ONLY.

- 2D image data can be input in the following formats:

filename.mat (where the file name is the same as the variable name), filename.gif, filename.bmp, or filename.jpg.

Data format:

- 2D image data can be entered.

- 3D Kspace data can be input. If PEs, FEs and num_coils are the number of

phase encodings, frequency encodings and number of coils, then data can be

entered as

PEs X FEs X num_coils or

FEs X PEs X num_coils.

- Extra data, data from reference scans, etc. should be entered as mentioned    previously.

- Data files are added in the folder ‘workdir/examples/data’.

- If reduced data and calibration data are collected separately, then field of view

(FOV) must be accurately known for correct location of the calibration data.

E.g.

Actual FOV = 10 cm

Data size = 256 X 256

10cm ≈ 256 lines

1cm ≈ 25 lines (25.6 exact)

10/256 cm ≈ 1 line

If calibration data is collected from the 5th cm then the line location will be found as 5 cm -> 25.6 X 5 = line no. 128.

If the next line is at 5.04 cm, then the line no. is 129 and so on. These locations are stored in a vector and entered for the variable center_locs (Sampling).

3.2. Image Reconstruction Process

- The image reconstruction process is executed depending on the type of data input in the toolbox.

- The reconstructed image I_rec is available in the file parameters.mat

(located in workdir/source/parameters.mat) and the parameters in the structure

Perform_parameters.

To view this image, enter the following command in the command window.

>>imagesc(abs(I_rec));

To view the image in black and white or to view the color scale, enter the following:

>>colormap(‘gray’);

>>colorbar;

3.3. Performance Evaluation

The method of reconstruction is analyzed on the basis of a number of parameters selected during the program.

- For SNR Calculation - Method 3, two acquisitions are necessary. User will be

prompted to enter data twice. For SNR Calculation - Method 2, at least two

acquisitions are required. For approximate results, 7 to 10 acquisitions are

necessary. Otherwise, at least 50 reconstructions are required. Each acquisition is

saved separately.

- For the Artifact Power Calculation, a reference image must be available.

- For SNR and CNR calculations, user must define the area of signal/region of

interest and the area for considering noise when prompted.

- For checking resolution, the resolution phantom must be selected.  Hence, it is

available only as a simulation result.

Some resolution phantoms are provided. A ray phantom and a square phantom are generated by rays_image.m and res_image.m, respectively.

- If SNRbox, Artifact_Pwr, box_snr are used as a stand-alone command (i.e. not called through variable per_list), the return value is stored in a temporary variable ans.

E.g.

1. >>SNRbox(I_rec)

SNR value is stored in ans

2. >>snr_value=SNRbox(I_rec)

SNR value is stored in snr_value

- If per_list is set to 1 in Input_list.m, the value will be displayed in the command window.  Otherwise, the value can be recalled by the following commands:

>>load parameters

>>Perform_parameters

4. Control Variables

The various inputs that can be fed to the program are listed alphabetically in the following table. Not all are required for a particular simulation. Parenthesis indicate the structure including the variable. Most of the inputs are assigned to a structure. The structures are:

Files

Image_size

Receiver

Sampling

Flags

Misc

E.g.

auto_smash_flag (Flags): referred as Flags.auto_smash_flags

4.1. Variables

| Variable/ Inputs | Type | Range | Description |
| --- | --- | --- | --- |
| auto_smash_flag(Flags) | Integer | 0-1 | 0: Perform Regular SMASH Reconstruction 1: Perform AUTO-SMASH Reconstruction |
| backgroundphase_freq (Receiver) | Float |  | For phase variations in the sensitivity estimation |
| blocks (Misc) | Integer |  | Choose no. of blocks for GRAPPA reconstruction |
| calibration (Files) | Text |  | File containing central data for sensitivity estimation |
| center_lines(Sampling) | Integer |  |  |
| center_locs (Sampling) | Vector of Integers |  | Vector with locations of the center lines for self calibration. |
| coils | 2 X 1 matrix |  | Number of coils in the array in the Z and X direction |
| depth | float |  | depth |
| data_type | Integer | 1 – 4 | 1: Complete simulation, Image will be selected from the software package itself. 2: Use file containing image as input 3: Use full scan kspace data entered as a file 4: Use reduced scan kspace data as a file |
| direction (Sampling) | Text | X, Y | Direction of phase encoding/sub-sampling X: Sub-sampling along columns Y: Sub-sampling along rows |
| FEs (Image_size) | Integer |  | Total frequency encodings |
| fit_img (Flags) | Integer | 0-1 | 0: Do not fit harmonics tightly in SMASH 1: Fit harmonics tightly in SMASH |
| lin_flag (Flags) | Integer | 0-1 | 0: Non-linear array 1: Linear array |
| max_sensitivity (Receiver) | Integer |  | Maximum sensitivity for the simulated receivers |
| num_coils(Receiver) | Integer |  | Number of coils |
| option | Integer | 1-5 | 1: SENSE 2: PILS 3: SMASH 4: GRAPPA 5: SPACE RIP |
| PEs (Image_size) | Integer |  | Total phase encodings |
| per | Float | 1-100 | Percentage of FOV the object occupies |
| per_list | Integer | 1-6 | 1: SNR Method 1 2: Pixel wise SNR 3: SNR 2 acquisition 4: Artifact Power 5: Resolution 6: No evaluation |
| poly_tick (Flags) | Integer | 0-1 | 0:Do not perform polynomial filtering 1: Performa polynomial filtering |
| psi | Matrix |  | Noise correlation matrix |
| recons (Misc) | Integer |  | No. of reconstructions in GRAPPA |
| reference_scan (Files) | Text |  | File containing reference scans |
| reference_image (Files) | Text |  | File containing reference body coil image |
| regularization_flag (Flags) | Integer | 0-1 | 0: No regularization for SENSE 1: Regularization for SENSE |
| samp (Misc) | Integer | 1-4 | 1: Uniform lines 2: All lines in the center 3: Alternate lines in center 4: Non-uniform sampling 5: Use extra lines |
| sampling_location (Sampling) | Vector of integers |  | Vector containing the positions of the acquired lines |
| sensitivity_estimation (Receiver) | Integer | 1-3 | 1: Use reference scans and reference image 2: Self calibration using SOS method 3: Self calibration using singular value decomposition. |
| sos_flag (Flags) | Integer | 0-1 | 0: No iterative SOS Reconstruction 1: Iterative SOS Reconstruction |
| SNR (Receiver) | Float |  | Reference SNR for adding noise in a simulation |
| subsampling_factor(Sampling) | Integer |  | Reduction factor |
| variance_factor (Sampling) | Integer | 1-5 | Choose localization of the coil 1: Very localized 2: Not localized |

The following table lists the variables required for different types of data input.

| Variable/ Inputs | Simulation | Image | Full kspace data | Reduced kspace |
| --- | --- | --- | --- | --- |
| auto_smash_flag(Flags) |  |  |  |  |
| backgroundphase_freq (Receiver) |  |  |  |  |
| blocks (Misc) |  |  |  |  |
| calibration (Files) |  |  |  |  |
| center_lines(Sampling) |  |  |  |  |
| center_locs (Sampling) |  |  |  |  |
| coils |  |  |  |  |
| data_type | 1 | 2 | 3 | 4 |
| depth |  |  |  |  |
| direction (Sampling) |  |  |  |  |
| FEs (Image_size) |  |  |  |  |
| fit_img (Flags) |  |  |  |  |
| lin_flag (Flags) |  |  |  |  |
| max_sensitivity (Receiver) |  |  |  |  |
| mcmli_flag (Flags) |  |  |  |  |
| mcmli |  |  |  |  |
| num_coils(Receiver) |  |  |  |  |
| option |  |  |  |  |
| PEs (Image_size) |  |  |  |  |
| perc |  |  |  |  |
| per_list |  |  |  |  |
| poly_tick (Flags) |  |  |  |  |
| psi |  |  |  |  |
| recons (Misc) |  |  |  |  |
| reference_scan (Files) |  |  |  |  |
| reference_image (Files) |  |  |  |  |
| regularization_flag (Flags) |  |  |  |  |
| samp (Misc) |  |  |  |  |
| sampling_location (Sampling) |  |  |  |  |
| sensitivity_estimation (Receiver) |  |  |  |  |
| SNR (Receiver) |  |  |  |  |
| subsampling_factor(Sampling) |  |  |  |  |
| variance_factor (Sampling) |  |  |  |  |

5. Individual Subroutines

Individual programs, their inputs and outputs are described in the following table:

| Functions/ Subroutine | Description | Input | Output |
| --- | --- | --- | --- |
| arbitrarywire.m | Finds the corners of the coil loop to find magnetic field component along X ,Y, Z | Point on the grid where magnetic field has to be evaluated( x,y,z co-ordinates), end points of the wire | Magnetic field B at a point |
| addnoise.m | Adds noise in the simulated kspace data | Reference SNR SNR, ideal kspace data full_kspace_data | Noise kspace data |
| Artifact_Pwr.m | Calculates Artifact Power | Reference image, Reconstructed image I_rec | Artifact Power |
| auto_smash.m | AUTO_SMASH Reconstruction | data_type, full/reduced kspace data*, sampling_location, PEs, FEs | Reconstructed Image, I_rec |
| box_snr.m | Calculates SNR with 2 acquisition | Reconstructed images of 2 acquisitions | SNR |
| Bxarbwire.m Byarbwire.m Bzarbwire.m | magnetic field by analytical solution for magnetic field component along X-axis, along Y-axis, along Z-axis | Point on the grid where magnetic field has to be evaluated( x,y,z co-ordinates), length, direction and center of wire | X,Y and Z component of the B field |
| Error_chk.m | Checks if any parameters is entered incorrectly | All parameters | Error flag set to 1 if error present |
| estimate_maps.m | Estimates sensitivity profiles using SOS method | Full kspace data*, extra calibration data, sampling locations, data_type | Sensitivity maps, Regularization image** |
| evaluate_method.m | Main listing of performance analysis | data_type, per_list, flag for multiple/two acquisitions (two_acq), reconstructed image and files containing original data (data entered) | Value of performance parameters selected |
| field_map.m | Calculates the field map due to coil array | Aperature,FOV, Pix, Plane, offset, array_offset, theta, coils | Field map sensitivity |
| fitcurve.m, fitcurve_gauss.m | Gaussian fitting for SMASH and PILS | Sensitivity maps | Gaussian profiles, centers of coils |
| gencol_gen.m | Collected the necessary lines to regenerate a specific missed line | ACS, reduced kspace data with acquired lines at their proper locations (size= actual image size X num_coils), PES, FEs, start of the ACS lines-1, coil number (under reconstruction), num_coils, column number, blocks blocks, reconstructions recons, coils considered for generating coefficients consider_coils. | Correct column of acquired lines to generate the missed line |
| genmat_gen.m | Generated matrices for obtaining coefficients for GRAPPA reconstruction | Same as gencol_gen.m | Matrix of rearranged ACS lines for calculating coefficients |
| get_contour.m | Generates mask to extract object for polynomial filtering | Coil profile, threshold | Mask for profile |
| get_kspace_data.m | Generates reduced kspace data, computes sampling location | data_type, Files containing data, subsampling factor, number of receivers, localization factor, linear array |  |
| grappa.m | GRAPPA reconstruction | reduced kspace data with acquired lines at their proper locations (size= actual image size X num_coils), number of ACS lines, location to start, sampling_location, consider_coils, blocks, recons | I_rec |
| idft.m | Computes inverse DFT | Fourier data, Size | IDFT of data entered |
| Inputs_list.m | List of user definable parameters -to be modified as per need |  |  |
| kspacedata_sim.m | Simulates k-space data for a simulation | Image image, coil sensitivities, subsampling_factor | Full kspace data, reduced kspace data, sampling_location |
| make_profile, make_gauss_profile | fitting for SMASH and PILS, call fitcurve.m, fitcurve_gauss.m | Sensitivity maps | Gaussian profile. Centers of coils |
| myfun.m | Contains Gaussian function used for approximation |  |  |
| non_linear_array.m | Generate sensitivities for a non-linear array | image, num_coils, variance_factor | Sensitivity profiles |
| pils_gauss.m | PILS reconstruction | rough_map, reduced_kspace_data, lin_flag, subsampling_factor | Reconstructed image I_rec |
| pixel_snr.m | calculates pixel-wise SNR | array containing reconstructed images from all acquisitions, loop_cnt | SNR map |
| PMRI_Toolbox.m | GUI to use software |  |  |
| prepare_grappa2.m | Collects ACS lines and aligns them for GRAPPA reconstruction |  |  |
| profile_1pd.m | Generates sensitivity profiles for the 1D Gaussian linear array | Coil localization, PEs, FEs, Coil centers background phase frequency | Coil profiles |
| rays_image.m | Generates the ray phantom for resolution check | - | image |
| sample_kd.m | Decimates full kspace data | full_kspace_data, subsampling_factor | reduced_kspace_data, sampling_location |
| sense.m | SENSE reconstruction | reduced_kspace_data, rough_map, psi | I_rec |
| sense_tikhnov.m | Regularized SENSE reconstruction | reduced_kspace_data, rough_map, regularization_image, psi | I_rec |
| sensitivity_ estimation_ process.m | Main program to call the selected the correct sensitivity estimation process | data_type, option, full_kspace_data, reduced_kspace_data, Receiver, Misc, Sampling, Flags, Image_size, per_list | rough_map, regularization_image |
| sensitivity_ polyfilt.m | Polynomial filtering for sensitivity estimation | Noisy sensitivities, Order of the filter, and Neighborhood | Filtered sensitivities |
| smash.m | SMASH reconstruction | reduced_kspace_data, profiles, subsampling_factor, sampling_location, fit_img(Flags), perc (Percentage FOV occupied by object) | I_rec |
| smash_fit_harmonic.m | Generates coefficients for harmonics | profiles, subsampling_factor, sampling_location, fit_img(Flags), perc (Percentage FOV occupied by object) | Smooth harmonics (sine and cosine parts), SMASH coefficients |
| SNRbox | Calculates SNR by method 1 | I_rec | SNR |
| sos_recon.m | Iterative SOS reconstruction | I_rec, center_data |  |
| spacerip.m | SPACE RIP reconstruction | reduced_kspace_data, rough_map, sampling_location | I_rec |
| square_loop_B.m | Calculates the corners of the wire loop to find the effect of each wire individually at a point | Length and width of each coil, centers of each coil, grid of points to evaluate B | Magnetic field map B |
| subsamp_gen.m | Non-uniform sampling for SPACE RIP | data_type, full_kspace_data, samp, total number of lines acquired, subsampling_factor | reduced_kspace_data, sampling_location |
| walsh_sensitivity.m | Sensitivity estimation using SVD | full_kspace_data, center_data, center_lines, subsampling_factor, num_coils, PEs, FEs, data_type | Sensitivity maps, Regularization image** |

* If only reduced kspace data is available, full kspace data is set to 0.

** Regularization image for Regularized SENSE otherwise = 0.

6. Structures

To use the software tool as a whole, variables are passed in the form of ‘structures’ between functions. For running or using individual programs, the same programs are available in the folder individual codes.  The programs are the same and have the same name but variables are passed individually.

E.g.

In the entire tool box, the reduction factor is called subsampling_factor in the structure Sampling. This structure will also contain other variables. To use the variable, it must be referred to as Sampling.subsampling_factor. It is passed between functions within the structure Sampling, not as an individual variable. In independent programs, the function will pass the variable as subsampling_factor only. The exact variable needed for the execution of the function can be easily found in the individual codes. Otherwise, the entire structure is passed and the variables from the structure that will actually be used cannot be clearly understood.