PULSAR Version 1.00

Parallel Imaging Utilizing Localized Surface-coil Acquisition and Reconstruction

USER MANUAL for PILS

Magnetic Resonance Systems Lab

Department of Electrical Engineering

Texas A&M University

COPYRIGHT

Copyright 2005, Department of Electrical Engineering, Texas A&M University.

All rights reserved. See the file COPYRIGHT for more information.

You are granted a license to use the Software for academic, research and internal business purposes only, without a fee.  You may distribute the software to third parties provided that the copyright notice and this statement appear on all copies and that no charge is associated with such copies. If you make derivative works and distribute it, you will (1) notify PULSAR developers regarding its distribution of the derivative work, and (2) clearly notify users that such derivative work is a modified version and not the original PULSAR distributed by Texas A&M University.

We MAKE NO REPRESENTATIONS ABOUT THE SUITABILITY OF THIS SOFTWARE FOR ANY PURPOSE.  IT IS PROVIDED "AS IS" WITHOUT EXPRESSED OR IMPLIED WARRANTY. WE SHALL NOT BE LIABLE FOR ANY DAMAGES SUFFERED BY THE USERS OF THIS SOFTWARE.

Any Licensee wishing to make commercial use of the Software should contact:

Jim Ji

Department of Electrical Engineering

Texas A&M University

College Station, TX 77843-3128

jimji@tamu.edu

PULSAR TEAM:

Jim X. Ji, Jong Bum Son, Swati Rane, Dan Spence, Leslie Chan

INDEX

Chapter 1. Introduction……………………………………………………………….….…...4

Chapter 2. System Requirements………………………………………………….………….4

Chapter 3. Installing PULSAR for PILS……………………...……….……………….….....4

Chapter 4. Getting Started…………………………………………...……….………….…....5

Chapter 5. Tutorials……………………………...……………….……………….……….…. 6

Chapter 6. Contacts……………………………...……………….……………….……….…. 9

USER MANUAL

Jim X. Ji, Swati D. Rane and Jong Bum Son (Dante Son)

Department of Electrical Engineering, Texas A&M University

1. Introduction

This manual is written in order to explain how to use this software tool. This software package allows the user to simulate the parallel magnetic resonance imaging technique, Partially Parallel Imaging with Localized Sensitivities (PILS). For specific information on this technique, please refer to the paper:

▪ Partially parallel imaging with localized sensitivities (PILS)

Magnetic Resonance in Medicine

Volume 44, Issue 4, Date: October 2000, Pages: 602-609

Mark A. Griswold, Peter M. Jakob, Mathias Nittka, James W. Goldfarb, Axel Haase

User can learn how to run the software package easily through the quick-start guide.

2. System Requirements

The software package requires:

▪ MATLAB 6.5 or higher

▪ WinZip 8.1 SR1 or higher

It requires equivalent system requirements of MATLAB 6.5 or higher version. For specific information, see the requirements, platforms and operating systems in the MathWorks website, www.mathworks.com/support.

MATLAB is a registered trademark of The MathWorks, Inc. (www.mathworks.com)

WinZip is a registered trademark of WinZip International LLC. (www.winzip.com)

3. Installing PULSAR for PILS

Installing PULSR for GRAPPA is very simple. Just extract ‘PULSAR.zip’ on your hard disk to a directory, e.g., ‘c:\PULSAR’.

4. Getting Started

The software package has built-in 4-channel k-space data for demo. In order to try the demo, please follow:

a. Start MATLAB 6.5 or higher.

b. Move to the ‘c:\PULSAR\source\pils’ directory

c. Open ‘startup.m’ with any text editor and set the path according to the current directory:

Example) 	path(path, 'C:\PULSAR\source\common');

path(path, 'C:\PULSAR \source\pils');

d. Type ‘recon’ on a command window.

e. After processing, reconstructed image is displayed on the screen.

Figure 1. Getting Started Demo

5. Tutorial

(1) Overview

‘Pulsar for PILS’ consists of four files: three m-files and one built-in data file for demo. The entire procedure consists of two steps: sensitivity estimation and image reconstruction with PILS. Each step is implemented in an independent function in order to facilitate full customization.

recon.m

▪ Main m-script

▪ Data read-in

Data_spin.mat

▪ 4-ch full k-space data

‘reduced_kspace_data’ : subsampled k-space data

‘central_kdata’     : additionally acquired full PE lines for estimating the coil center locations required by PILS algorithm

sensitivity_estimation_forpils.m

▪ Control variable setting

▪ Coil center location with Gaussian-fitting

‘reduced_kspace_data’  : subsampled k-space data

‘coil_center_locations’   : control variable

pils.m

▪ PILS reconstruction

‘I_recon’: reconstructed image

(For the structure, see the Table 3)

recon.m

▪ Image display

Red color : Matix name for data storage

: Flow of data or the direction of reference

: M-file name

: Functions of m-file

Figure 2. The overview of PULSAR for GRAPPA

(2) Specifications

A. recon.m

▪ Main m-script

▪ Initialization, setup FOV reduction factor (Reduction factor), number of full-PE lines

▪ Data read-in for subsampled k-space data and additionally acquired k-space lines for coil-sensitivity estimation

▪ Image display

B. data.mat

| Matrix Name | Structure | Description |
| --- | --- | --- |
| Full_kspace_data | Nfe x Npe x Ncoil Nfe : Number of frequency encoding Npe: Number of phase encoding R : Number of phase encoding Ncoil:Number of coil | Full 4ch spine k-space data |

Table 1. Input/Output Specifications

C. sensitivity_estimation.m

▪ Generation of low-resolution image for coil-sensitivity estimation

When partially reconstructed images from each coil are combined together in order to compose a final reconstructed image, PILS refers to the central location of each coil in the image domain. In order to estimate the location of coil center, low resolution images for each coil are reconstructed first using additionally acquired k-space lines.

▪ Coil-sensitivity estimation with Gaussian-fitting

Coil locations can be successfully estimated with function-fitting. In ‘PULSAR for PILS,’ Gaussian model is used for coil-sensitivity distribution.

| Input / Output | Control Variable(CV) or Data | Description |
| --- | --- | --- |
| input | reduced kspace_data | Subsampled k-space data |
| input | center_kdata | Additionally acquired k-space data for coil-sensitivity estimation |
| input | FOV_reduction_factor | Subsampling factor |
| output | Coil_center_location | Estimated location of coil-center (fe,pe) |

Table 2. Input/Output Specifications

D. pils.m

▪ PILS reconstruction

| Input / Output | Control Variable(CV) or Data | Description |
| --- | --- | --- |
| input | reduced kspace_data (Nfe x (Npe / R) x Ncoil) Nfe : Number of frequency encoding Npe: Number of phase encoding R : Number of phase encoding Ncoil:Number of coil | Subsampled k-space data |
| input | FOV_reduction_factor | R, subsampling factor |
| input | Coil_center_location | Coil center, in example, it’s 1D |
| output | I_recon (structure: Nfe x Npe) Nfe: Number of frequency encoding Npe: Number of phase encoding | Reconstructed image with PILS |

Table 3. Input/Output Specifications

6. Contacts

Please respond for bug report and your opinions to:

Parallel.mri@gamil.com