PULSAR Version 1.00

Parallel Imaging Utilizing Localized Surface-coil Acquisition and Reconstruction

USER MANUAL for SPACE-RIP

Magnetic Resonance Systems Lab

Department of Electrical Engineering

Texas A&M University

COPYRIGHT

Copyright 2005, Department of Electrical Engineering, Texas A&M University.

All rights reserved. See the file COPYRIGHT for more information.

You are granted a license to use the Software for academic, research and internal business purposes only, without a fee.  You may distribute the software to third parties provided that the copyright notice and this statement appear on all copies and that no charge is associated with such copies. If you make derivative works and distribute it, you will (1) notify PULSAR developers regarding its distribution of the derivative work, and (2) clearly notify users that such derivative work is a modified version and not the original PULSAR distributed by Texas A&M University.

We MAKE NO REPRESENTATIONS ABOUT THE SUITABILITY OF THIS SOFTWARE FOR ANY PURPOSE.  IT IS PROVIDED "AS IS" WITHOUT EXPRESSED OR IMPLIED WARRANTY. WE SHALL NOT BE LIABLE FOR ANY DAMAGES SUFFERED BY THE USERS OF THIS SOFTWARE.

Any Licensee wishing to make commercial use of the Software should contact:

Jim Ji

Department of Electrical Engineering

Texas A&M University

College Station, TX 77843-3128

jimji@tamu.edu

PULSAR TEAM:

Jim X. Ji, Jong Bum Son, Swati Rane, Dan Spence, Leslie Chan

INDEX

Chapter 1. Introduction……………………………………………………………….….…...4

Chapter 2. System Requirements………………………………………………….………….4

Chapter 3. Installing PULSAR for SPACE-RIP………………...………….………….….....4

Chapter 4. Getting Started…………………………………………...……….………….…....5

Chapter 5. Tutorials……………………………...……………….……………….……….…. 6

Chapter 6. Contacts……………………………...……………….……………….……….…. 10

USER MANUAL

Jim X. Ji, Swati D. Rane and Jong Bum Son (Dante Son)

Department of Electrical Engineering, Texas A&M University

1. Introduction

This manual is written in order to explain how to use this software tool. This software package allows the user to simulate the parallel magnetic resonance imaging technique, ‘Sensitivity Profiles from an Array of Coils for Encoding and Reconstruction In Parallel (SPACE RIP)’. For specific information on this technique, please refer to the paper:

▪ SENSE Sensitivity Profiles from an Array of Coils for Encoding and Reconstruction In Parallel (SPACE RIP), Walid E. Kyriakos, Lawrence P. Panych, Daniel F. Kacher, Carl-Fredrick Westin, Sumi M. Bao, Robert V. Mulkern, Ferenc A. Jolesz, Magnetic Resonance in Medicine Volume 44, Issue 2, Date: August 2000, Pages: 301-308

User can learn how to use the software package easily through the quick-start guide.

2. System Requirements

The software package requires:

▪ MATLAB 6.5 or higher

▪ WinZip 8.1 SR1 or higher

It requires equivalent system requirements of MATLAB 6.5 or higher version. For specific information, see the requirements, platforms and operating systems in the MathWorks website, www.mathworks.com/support.

MATLAB is a registered trademark of The MathWorks, Inc. (www.mathworks.com)

WinZip is a registered trademark of WinZip International LLC. (www.winzip.com)

3. Installing PULSAR for SPACE-RIP

Installing PULSR for SPACE-RIP is very simple. Just extract ‘PULSAR for SPACE-RIP.zip’ on your harddisk. The folder, ‘PULSAR for SPACE-RIP,’ is automatically generated and all the files are installed there.

4. Getting Started

The software package has built-in 8-channel k-space data for demo. The subsampling factor for this demo is four. In order to try the demo, please follow:

a. Start MATLAB 6.5 or higher.

b. Move to the ‘c:\PULSAR\source\space-rip’ directory

c. Open ‘startup.m’ with any text editor and please set up the path:

Example) 	path(path, 'C:\PULSAR\source\common');

path(path, 'C:\PULSAR \source\space-rip');

d. Type ‘recon’ on a command window.

e. After processing, reconstructed image is displayed on the screen.

Figure 1. Getting Started Demo

5. Tutorial

(1) Overview

‘data1’: 8-channel full k-space data for demo

(For the data structure, see the Table 2)

The entire procedure consists of three steps: k-space data read-in, sensitivity estimation and SPACE-RIP reconstruction.

recon.m

▪ Main m-script

data1.mat

▪ 8 channel k-space data

‘Filename’: The filename for SENSE calibration

‘datatype’: Data type for additionally acquired k-space lines

‘Npe_tobe’: The phase encoding number without subsampling

‘Filters’: The filter definitions for sensitivity estimation

‘Rnoise’: Noise covariance matrix of the phased-array coils

(For the structure, see the Table 4)

‘full_kspace_data’: full k-space data with multiple channels

‘Wr_tobe’: Desirable number of rows after cropping

‘Wc_tobe’: Desirable number of columns after cropping

(For the data structure, see the Table 1)

datacrop2d.m

▪ Crop data to keep central k-space

‘full_kspace_data’: Chopped full k-space data

‘FOV_reduction_factor’: Subsampling factor

(For the data structure, see the Table 3)

sensitivity_estimation.m

8chcentralkdata.mat

‘coilimages’: Additionally acquired

central k-space data during

SENSE calibration for sensitivity

estimation (For the data structure, see the Table 5)

sample_kd.m

▪ Sensitivity estimation with filtering or fitting

▪ Subsample k-space data

‘reduced_kspace_data’: Subsampled k-space data

‘Subsampling_locations’: Locations for sub-sampled data

(For the structure, see the Table 6)

spacerip.m

‘channelsensitivity’: Estimated sensitivity

(For the structure, see the Table 4)

▪ SPACE-RIP reconstruction

▪ Additionally acquired

central k-space data

‘recon’   : Reconstructed image with SPACE-RIP

‘sucessful_flag’: 1 if reconstruction is successful

(For the structure, see the Table 6)

Red color 	: Matix name for data storage

: Flow of data or the direction of reference

: M-file name

: Functions of m-file

Figure 2. The overview of PULSAR for SPACE-RIP

(2) Specifications

A. recon.m

▪ Main m-script

▪ Initialization

▪ Image display

B. data1.mat

▪ 8 channel k-space data

| Matrix Name | Structure | Description |
| --- | --- | --- |
| data1 | Nfe x Npe x Ncoil Nfe: Number of frequency encoding Npe: Number of phase encoding Ncoil: Number of coils | Full k-space data with 8 channels |

Table 1. Data Structure of data1.mat

C. datacrop2d.m

▪ Crop data to keep central k-space

This function is to crop k-space in order to keep the central portion of data.

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | full_kspace_data | Full k-space data with multiple channels (structure: Nfe x Npe x Ncoil) Nfe: Number of frequency encoding Npe: Number of phase encoding Ncoil: Number of coils |
| input | Wr_tobe | Desirable number of rows after cropping |
| input | Wc_tobe | Desirable number of columns after cropping |
| output | full_kspace_data (cropped) | Chopped full k-space data (structure: Wr_tobee x Wc_tobee x Ncoil) |

Table 2. Input/Output Specifications of ‘datacrop2d.m’

D. sample_kd.m

▪ Subsample k-space data

This function simulates subsampled k-space data from each coil.  The direction of reduction is column-directional. (i.e. phase-encoding direction)

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | full_kspace_data | Chopped full k-space data (structure: Wr_tobee x Wc_tobee x Ncoil) |
| input | FOV_reduction_factor | Subsampling factor |
| output | reduced_kspace_data | Subsampled k-space data (structure: Wr_tobee x (Wc_tobee /FOV_reduction_factor) x Ncoil) |
| output | Subsampling_locations | Locations for sub-sampled data |

Table 3. Input/Output Specifications of ‘sample_kd.m’

E. sensitivity_estimation.m

▪ Sensitivity estimation with filtering or fitting

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | Filename | The filename for SENSE calibration. Default is ‘8chcentralkdata.mat’ |
| input | datatype | Data type for additionally acquired k-space lines Options: ‘1’: File contains central k-space data collected with a full FOV without aliasing. It should comes from the central portion of the k-space symmetrically from -L/2:1:(L/2-1) assuming total L lines. ‘2’: File contains low-resolution coil images, the FOV must be equal to reduced FOV times of FOV_reduction_factor. However, it can have arbitrary number of lines: the program will resample the image so it will have the correct number of lines (i.e., Npe_tobe = Npe_of_the_reduced_k-space_data * FOV_reduction_factor) ‘3’: File contains the sensitivity functions already estimated by measured elsewhere. |
| input | Npe_tobe | The phase encoding number without subsampling |
| input | Filters | The filter definitions for sensitivity estimation Options for ‘Filters.type’: 'nofilter' : no filter applied 'polynomial': Polynomial fitting is used for coil-sensitivity estimation. Additionally, ‘Filters.Poly_Order’ needs to be set up to select the order of polynomial. ‘Filters.poly_Wr’ and ‘Filters.poly_Wc’ represent the size of polynomial filtering along row and column for each. 'median' : Medial filter is used for coil-sensitivity estimation 'wavelet' : Wavelet filter is used for coil-sensitivity estimation. Additionally, ‘Filters.wavelet_name’ and ‘filters.wavelet_levels‘ need to be set up to select a wavelet model and level. 'hamming' : Hamming filter is used for coil-sensitivity estimation 'walsh' : Singular value decomposition is used for coil-sensitivity estimation. Additionally, ‘Filters.walsh_width’ needs to be set up the size of averaging window. |
| input | Rnoise | Noise covariance matrix of the phased-array coils (structure: Ncoil x Ncoil) |
| input | coilimages | Additionally acquired central k-space data for coil-sensitivity estimation (structure: Wr_tobee x Wc_tobee x Ncoil) |
| output | channelsensitivity | Estimated sensitivity for each coil (structure: Wr_tobee x Wc_tobee x Ncoil) |
| output | ROI_mask | Binary mask indicating ROI (structure: Wr_tobee x Wc_tobee) |

Table 4. Input/Output Specifications of ‘sensitivity_estimation.m’

F. 8chcentralkdata.mat

▪ Additionally acquired central k-space data for coil-sensitivity estimation

| Matrix Name | Structure | Description |
| --- | --- | --- |
| coilimages | NWr_tobe x NWc_tobe x Ncoil NWr_tobe: Number of chopped frequency encoding lines NWc_tobe: Number of chopped phase encoding lines Ncoil: Number of coils | Additionally acquired central k-space data for coil-sensitivity estimation |

Table 5. Data structure of ‘8chcentralkdata.mat’

G. spacerip.m

▪ SPACE-RIP reconstruction

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | reduced_kspace_data | Subsampled k-space data possibly with variable density Two kinds of formats are allowed: Format 1) Reduced_ksapce_data can be a matrix with the same size as channelsensitivity: it has all phase-encoding lines, but only at the sampled locations it has data. The other lines are simply zeros. As a convention, the central k-space is at (Nfe/2+1, Npe/2+1). (structure: Wr_tobe x Wc_tobe x Ncoil) Format 2) reduced_kspace_data can contain only the subsampled lines. In this case, the index of these lines in the full-FOV data must be provided, using a third input, Subsampling_locations (structure: Wr_tobe x (Wc_tobe /FOV_reduction_factor ) x Ncoil) |
| input | channelsensitivity | Spatial coil sensitivity function. It is required that ‘phase-encoding numbers without subsampling’ is equal to ‘subsampled phase-encoding lines’ times of ‘Subsampling factor.’ To enforce this condition, the coil sensitivity function needs to be interpolated with zero-padding k-space (structure: Wr_tobe x Wc_tobe x Ncoil) |
| input | tolerance | In solving SPACE-RIP matrix using SVD, any singular value less than ‘tolerance * max(eigen values)’ is treated as zero. Usually zero value of tolerance is used. A larger value improves the numerical condition, but may result in more residual aliasing errors. (Range of value: 0.0 ~ 1.0) |
| input | Subsampling_locations | Locations for sub-sampled data |
| output | recon | Reconstructed image with SPACE-RIP (structure: Wr_tobe x Wc_tobe) |
| output | successful_flag | 1 if reconstruction is successful |

Table 6. Input/Output Specifications of ‘spacerip.m’

6. Contacts

Please respond for bug report and your opinions to:

Parallel.mri@gamil.com