PULSAR Version 1.00

Parallel Imaging Utilizing Localized Surface-coil Acquisition and Reconstruction

USER MANUAL for GRAPPA

Magnetic Resonance Systems Lab

Department of Electrical Engineering

Texas A&M University

COPYRIGHT

Copyright 2005, Department of Electrical Engineering, Texas A&M University.

All rights reserved. See the file COPYRIGHT for more information.

You are granted a license to use the Software for academic, research and internal business purposes only, without a fee.  You may distribute the software to third parties provided that the copyright notice and this statement appear on all copies and that no charge is associated with such copies. If you make derivative works and distribute it, you will (1) notify PULSAR developers regarding its distribution of the derivative work, and (2) clearly notify users that such derivative work is a modified version and not the original PULSAR distributed by Texas A&M University.

We MAKE NO REPRESENTATIONS ABOUT THE SUITABILITY OF THIS SOFTWARE FOR ANY PURPOSE.  IT IS PROVIDED "AS IS" WITHOUT EXPRESSED OR IMPLIED WARRANTY. WE SHALL NOT BE LIABLE FOR ANY DAMAGES SUFFERED BY THE USERS OF THIS SOFTWARE.

Any Licensee wishing to make commercial use of the Software should contact:

Jim Ji

Department of Electrical Engineering

Texas A&M University

College Station, TX 77843-3128

jimji@tamu.edu

PULSAR TEAM:

Jim X. Ji, Jong Bum Son, Swati Rane, Dan Spence

INDEX

Chapter 1. Introduction……………………………………………………………….….…...4

Chapter 2. System Requirements………………………………………………….………….4

Chapter 3. Installing PULSAR for GRAPPA……………………...……….………….….....4

Chapter 4. Getting Started…………………………………………...……….………….…....5

Chapter 5. Tutorials……………………………...……………….……………….……….…. 7

Chapter 6. Contacts……………………………...……………….……………….……….…. 9

USER MANUAL

Jim X. Ji, Swati D. Rane and Jong Bum Son (Dante Son)

Department of Electrical Engineering, Texas A&M University

1. Introduction

This manual is written in order to explain how to use this software tool. This software package allows the user to simulate the parallel magnetic resonance imaging technique, GRAPPA(GeneRalized Autocalibrating Partially Parallel Acquisition). For specific information on this technique, please refer to the papers:

▪ Generalized Autocalibrating Partially Parallel Acquisitions (GRAPPA), M. Griswold, Magnetic Resonance in Medicne, 47:1202-1210 (2002)

User can learn how to run the software package easily through the quick-start guide.

2. System Requirements

The software package requires:

▪ MATLAB 6.5 or higher

▪ WinZip 8.1 SR1 or higher

It requires equivalent system requirements of MATLAB 6.5 or higher version. For specific information, see the requirements, platforms and operating systems in the MathWorks website, www.mathworks.com/support.

MATLAB is a registered trademark of The MathWorks, Inc. (www.mathworks.com)

WinZip is a registered trademark of WinZip International LLC. (www.winzip.com)

3. Installing PULSAR for GRAPPA

Installing PULSR for GRAPPA is very simple. Just extract ‘PULSAR.zip’ on your local folder. The subfolder, ‘c:\PULSAR,’ is automatically generated and all the files are installed there.

4. Getting Started

The software package has built-in 4-channel spine and 8-channel brain in-vivo images for demo. In order to try the demo, please follow the steps:

a. Start MATLAB 6.5 or higher.

b. Move to the ‘c:\PULSAR\source\grappa’ directory

c. Type ‘recon’ on a command window.

d. With a GUI, we select a MATLAB data-file(.mat extension), which is used for the data-input of ‘PULSAR for GRAPPA.’ Please select ‘data_spine’ or ‘data_brain’ and press the ‘Open’ button.

Note: The dimension of k-space data matrix in the file is [PE FECOIL]. The file should include only one matrix and the name of matrix in the file can be arbitrary.

e. For the question, ‘Subsampling factor? [default: 2]’ please enter your acceleration factor. The default value is ‘2’ when there is no user-input.

Note: Input range is 2  Subsampling factor  Number of coil.

f.  For the next question, ‘Block size? [default: 4]’ please enter the number of block, which is used for GRAPPA reconstruction. The block size of ‘4’ is used when user does not input any value for it.

Note: Input range is 1Block size.

g. For the question, ‘Number of ACS lines? [default: 32],’ please enter the number of additionally acquired k-space lines required for GRAPPA reconstruction.

Note: Input range is Block size Number of coils  Subsampling factor  Full phase-encoding lines.

h. The next question is ‘Number of coils for GRAPPA reconstruction? [default :4].’ We select the number of coils for GRAPPA reconstruction.

Note: Input range is 1  Subsampling factor  Number of coils.

i. We need to select the coil-type of geometric distribution for the question of ‘Type of coil distribution? 1: Linear, 2:circularly symmetric coils [default: 1].’ The 4-channel spine demo has linear distribution of coils. On the other hands, the 8-channel brain demo has circularly symmetric coils.

j. For the last question, the way of applying Fourier transform is determined. ‘PULSAR for GRAPPA’ supports two options for it.

Option 1: 1D IFFT along FE -> GRAPPA Reconstruction -> 1D IFFT along PE

Option 2: GRAPPA Reconstruction -> 2D IFFT

k. After processing, reconstructed image is displayed on the screen. The left image is the reconstructed image with PULSAR for GRAPPA and the right image shows the sum-of-square image for reference, composed by full k-space data.

Figure 1. Getting Started Demo

5. Tutorial

(1) Overview

The entire procedure of ‘PULSAR for GRAPPA’ consists of two steps: k-space subsampling and image reconstruction with GRAPPA. Each step is implemented in an independent function for easy customization.

‘kspace_data’: full k-space data

‘header’: control variable

(For the structure, see the Table 1)

recon.m

▪ Main m-script

data_spine.mat

▪ full k-space data

kspace_subsample.m

▪ Control variable setting

▪ Generation of subsampled k-space data

‘kspace_data’

: full k-space data

‘subsampled_kspace_data’: subsampled k-space data

‘header’: control variable

(For the structure, see the Table 2)

grappa.m

▪ GRAPPA reconstruction

‘I_recon’: reconstructed image

(For the structure, see the Table 3)

recon.m

▪ Image display

Red color : Matix name for data storage

: Flow of data or the direction of reference

: M-file name

: Functions of m-file

Figure 2. The overview of PULSAR for GRAPPA

(2) Specifications

A. recon.m

▪ Main m-script

▪ Initialization

▪ Image display

B. data_spine.mat

▪ Full k-space dataset for demo

| Matrix Name | Structure | Description |
| --- | --- | --- |
| kspace_data | Npe x Nfe x Ncoil Npe: Number of phase encoding Nfe: Number of frequency encoding Ncoil: Number of coil | Full k-space data for demo |

Table 1. Input/Output Specifications

C. kspace_subsample.m

▪ Control variable setting

▪ Generation of subsampled k-space data

‘PULSAR for GRAPPA’ basically asks users several essential control variables: ‘subsampling factor,’ ‘block size,’ ‘the number of ACS lines,’ and ‘the number of coils,’ ‘type of coil,’ ‘the way of applying Fourier transform.’ Because most of Control Variables(CV) are set-up automatically, for most cases, users do not need to set them up manually. However, if it is necessary, ‘PULSAR for GRAPPA’ supplies the way to customize them specifically.

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | kspace_data | Full k-space data |
| input | header. subsampling_factor | Acceleration factor for GRAPPA reconstruction |
| input | header.blocks | Number of reference blocks for GRAPPA reconstruction |
| input | header.central_ksapce | Number of ACS lines |
| input | header.consider_coils | Number of coils used for GRAPPA reconstruction |
| input | header.coil_type | Type of coil's geometric distribution 1: Linear, 2:circularly symmetric coils |
| input | header.fft | The method of applying Fourier transform 1: 1D IFFT along FE -> GRAPPA Recon -> 1D IFFT along PE 2: GRAPPA Recon -> 2D IFFT |
| output | header.Nfe | Number of frequency encoding lines |
| output | header.Npe | Number of phase encoding lines |
| output | header.num_coil | Number of coils |
| output | header.sampling_location | Index of subsampled PE and ACS lines |
| output | subsampled_kspace_data | Subsampled k-space data in the fashion of [1: header. subsampling_factor: header.Npe] |

Table 2. Input/Output Specifications

D. grappa.m

▪ GRAPPA reconstruction code consists of four parts: index calculation in order to use partial number of coils for GRAPPA reconstruction, index calculation for reference blocks, coefficient calculation using ACS lines for GRAPPA reconstruction, and the process of GRAPPA reconstruction using GRAPPA coefficients.

| Input / Output | Control Variable(CV) | Description |
| --- | --- | --- |
| input | subsampled_kspace_data (structure: Npe x Nfe x Ncoil) Nfe: Number of frequency encoding Npe: Number of phase encoding Ncoil: Number of coils Note: Not acquired k-space lines are filled with zero. | Subsampled k-space data Note: The position of subampled k-space data is rearranged to be located on the full k-space domain. Moreover, ACS lines are already incorporated. |
| input | header | Control variables |
| output | I_recon (structure: Npe x Nfe) Nfe: Number of frequency encoding Npe: Number of phase encoding | Reconstructed image with GRAPPA |

Table 3. Input/Output Specifications

6. Contacts

Please respond for bug report and your opinions to:

parallel.mri@gmail.com